1.	Download the latest version from the ALDOT website.

2.	Extract the files using the WinZip created .exe file to a folder of your choice but keep the existing directory structure intact. By default it will want to extract to c:\ so these instructions will reflect this. The files and folders will extract to a folder named ALDOT_Resources.

3.	The decision will need to be made whether to pull these resources from a server or install them locally. These instructions will attempt to guide the installer in either direction.

Local installation (for instructional purposes these instructions will be using the D partition)

4.	Copy or move the ALDOT_Resources directory to the D partition.

5.	Move ALDOT.ucf to the C:\ProgramData\Bentley\MicroStation V8i (SELECTseries)\WorkSpace\Users folder.

6.	Move ALDOT_Standards.pcf to the C:\ProgramData\Bentley\MicroStation V8i (SELECTseries)\WorkSpace\Projects folder.

7.	Open ALDOT_Standards.pcf for editing in Notepad. Change the line ALDOT_RESOURCES = //SERVERNAME/SHARENAME/ to ALDOT_RESOURCES = D:/ALDOT_Resources/

TO

8.	Open Microstation Manager and select ALDOT in the User pull down normally located on the bottom right.

This should also set the Project to ALDOT_Standards.

9.	Check that the resources are being read by opening a dgn and going to Workspace>Configuration, click on Cells in the left hand column and select Cell Library Directories from the list in the right hand pane. This should be reading from the location where the resources were placed.

Server installation (for instructional purposes these instructions will be using ALDOT for the sharename)

4.	Copy or move the ALDOT_Resources directory to a location on the server.

5.	Create a share of the ALDOT_Resources directory and name it ALDOT_Resources. (Does not matter what the share name is but it will need to be known later)

6.	Move ALDOT.ucf to the C:\ProgramData\Bentley\MicroStation V8i (SELECTseries)\WorkSpace\Users folder.

7.	Move ALDOT_Standards.pcf to the C:\ProgramData\Bentley\MicroStation V8i (SELECTseries)\WorkSpace\Projects folder.

8.	Open ALDOT_Standards.pcf for editing in Notepad. Change the line ALDOT_RESOURCES = //SERVERNAME/SHARENAME/ to ALDOT_RESOURCES = //MYSERVER/ALDOT_RESOURCES/

TO

9.	Open Microstation Manager and select ALDOT in the User pull down normally located on the bottom right.

This should also set the Project to ALDOT_Resources.

10.	Check that the resources are being read by opening a dgn and going to Workspace>Configuration, click on Cells in the left hand column and select Cell Library Directories from the list in the right hand pane. This should be reading from the location the resources were placed.